Distributed EV Charging Scheduling Under Time-Varying Electricity Prices: A Decomposition and ADMM-Based Approach

- Brief Description and Abstract

This project studies a distributed optimization problem for coordinated electric vehicle (EV) charging in a smart charging network. The main topic is how to schedule the charging power of multiple EVs over time in a computationally efficient and scalable way, while satisfying practical constraints such as transformer capacity limits, per-vehicle charging rate bounds, charging deadlines, and time-varying electricity prices. The objective is to reduce the total charging cost and peak demand, and at the same time improve charging fairness among users.

The motivation of this topic is that EV charging is naturally a large-scale optimization problem. When the number of vehicles becomes large, a fully centralized solution may suffer from high computational complexity, communication overhead, and low flexibility in distributed implementation. Therefore, this project will investigate whether decomposition-based optimization methods, especially dual decomposition and ADMM, can achieve performance close to the centralized optimum while enabling distributed computation.

The project will first formulate a centralized convex optimization model for coordinated EV charging. In the basic model, each EV has an arrival time, a departure time, an energy demand, and a charging power constraint. The network side has a global or local power limit, and the electricity price may vary across time slots. A fairness term may also be introduced so that the charging strategy does not heavily favor only a small portion of vehicles. Based on this formulation, the project will derive distributed solution methods. In particular, dual decomposition will be used to separate the coupling constraints through dual variables, and ADMM will be used to further study distributed coordination with better robustness and convergence behavior.

The report will review at least five related papers on EV charging optimization, distributed energy management, decomposition methods, and ADMM-based scheduling. Their formulations, assumptions, advantages, and limitations will be compared in a unified optimization framework. Based on this review, the project will provide a small extension by considering time-varying electricity prices together with a simple fairness-aware charging objective, and then compare three solution strategies: centralized optimization, dual decomposition, and ADMM.

The numerical part will be implemented in Python on synthetic charging scenarios with different numbers of EVs. The comparison will focus on total cost, peak load, convergence speed, and fairness. The expected outcome is to show that distributed methods can closely approach the centralized solution while being more scalable and more suitable for practical smart charging systems.